CO3-AT1

Q11. Analyze the effect of frequency variation on the performance of a quarter-wave transformer using simulated S-parameters.

Code:

clc;

clear;

close all;

% Parameters

Z0 = 50;              % Characteristic impedance of main line (Ohm)

ZL = 100;             % Load impedance (Ohm)

f0 = 10e9;            % Design frequency = 10 GHz

% Quarter-wave transformer impedance

Zt = sqrt(Z0 * ZL);

% Frequency range

f = linspace(5e9, 15e9, 1000);

% Electrical length of transformer

theta = (pi/2) * (f/f0);

% Input impedance of quarter-wave transformer

Zin = Zt .* (ZL + 1i*Zt.*tan(theta)) ./ ...

(Zt + 1i*ZL.*tan(theta));

% Reflection coefficient

Gamma = (Zin - Z0) ./ (Zin + Z0);

% Return Loss

RL = -20*log10(abs(Gamma));

% VSWR

VSWR = (1 + abs(Gamma)) ./ (1 - abs(Gamma));

% Plot Return Loss

figure;

plot(f/1e9, RL, 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss vs Frequency');

% Plot VSWR

figure;

plot(f/1e9, VSWR, 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('VSWR');

title('VSWR vs Frequency');

% Display transformer impedance

fprintf('Quarter-wave Transformer Impedance = %.2f Ohm\n', Zt);

% Values at design frequency

[~, index] = min(abs(f-f0));

fprintf('At Design Frequency = %.2f GHz\n', f(index)/1e9);

fprintf('Reflection Coefficient = %.4f\n', abs(Gamma(index)));

fprintf('Return Loss = %.2f dB\n', RL(index));

fprintf('VSWR = %.2f\n', VSWR(index));

Output: